# Laboratory Safety

## General Safety Rules

- Work in a laboratory only during regular, assigned period when an instructor is present, unless specific authorization has been given by the instructor to work in the laboratory at other times.

- Read carefully and observe fully all laboratory instructions.  In case there is any doubt about any procedure, check with your instructor.

- Learn the location and proper use of emergency showers, fire extinguishers, and eye wash stations.

- Avoid inhaling chemical vapors or gases.  Use fume hoods for hazardous materials.

- Immediately wash off and chemicals spilled on the skin with lots of water.  In case of a serious spill, remove contaminated clothing immediately and flush affected area with lots of water.

- Do not eat, smoke, or drink in the laboratory.  Do not bring any food items into the laboratory.

- Do not leave experiments in progress unattended without authorization.

- Keep working areas neat and clean at all times.

- Report all accidents to the instructor immediately.

## Personal Protective Equipment

- All persons working with hazardous chemicals should wear gloves.

- All persons working with chemicals that could be splashed in the eyes are required to wear safety goggles or glasses.

- Contact lenses should not be worn in lab when hazardous chemicals or vapors are being used.

- Because of the danger of broken glassware or spilled chemicals, covered shoes should be worn in the laboratory.  (No types of open toe shoes are permitted in labs.)

## Chemical Safety

- Almost every chemical, whether solid, liquid or gaseous, is poisonous to the human body to some degree. Always use proper caution when handling chemicals.

- Consult a physician if you are pregnant or have any other medical condition that might render you susceptible to exposure to the chemicals used in this laboratory.

- When handling chemicals, keep your hands away from your face, eyes and body until your hands have been washed thoroughly.

- Do not taste any chemical. Label every container so items can be identified.

- When diluting acids, ALWAYS POUR ACID INTO WATER SLOWLY.

- Do not pipet anything into the mouth.

## Waste Disposal

- Always treat laboratory glassware as if it were fragile.  If glassware breaks, do not pick broken glass up with your hands.  Use a broom and dustpan, then place pieces in the cardboard box labeled  “Glass Disposal Box.”

- Do not pour any chemicals down the drain.  The instructor will advise you how to use the proper waste containers.

- Discarded animals parts must be placed in a red cardboard “Biohazardous Waste” box.

- Discarded sharp items including: scalpels, dissecting pins, probes, and needles must be placed in a red, plastic “Sharps Box.”

## Safety Agreement

## Biology 113

I have read the Lab Safety Rules and procedures for the prevention of injuries in the laboratory, and I will observe them in my lab work.

INSTRUCTOR’S NAME:

STUDENT’S NAME (PRINT)

STUDENT’S SIGNATURE

DATE