Institute for Systems Biology Protocol

Direct Incorporation of Cy3/Cy5 During Reverse Transcription

Written by Anne Rosenwald and Todd Eckdahl; August 2003)

Materials Needed:

# Isolated and quantitated total RNA samples

Microarray slides (70-mer plus-strand oligomers)

RNase-free water

oligo dT primer (16- to 18-mer) at 1 g/ul

Coverslips, 22 x 40mm size from Corning

100 mM DTT (dithiothreitol)

low dTTP dNTP mix (10 mM each dATP, DCTP, dGTP, 1 mM dTTP)

Cy3-dUTP and Cy5-dUTP (1 mM each [separately])

3 M Ammonium Acetate, pH 5.2

100% Ethanol, 70% Ethanol

Superscript II Reverse Transcriptase, 5X first strand buffer

RNase A (4 mg/ml)

RNase H (2 unit/l)

# Reverse Transcription and Cy-dye Incorporation

- In duplicate, aliquot 50 g total RNA (one for each treatment)

  - already checked with denaturing agarose gel

  - quantitated with UV spectrophotometer

  - precipitated (eg. 1/10 volume 3M NaOAc pH 5.2, 2 volumes EtOH)

- To each tube, add 2.5 g oligo dT

- Adjust volume to 11 l with DEPC-treated H2O

- Heat to 75oC for 10 min

- Cool slowly to room temperature and spin down (Note:  keep at RT from this point on)

- Add the following in order:

  - 4 l Superscript first strand 5X buffer

  - 2 l DTT (100 mM)

  - 1 l dNTPs (10 mM each dATP, dCTP, cGTP and 1 mM dTTP)

  - 1 l Cy-dye labeled dUTP (1 mM) (One gets Cy-3 dUTP and one gets Cy-5 dUTP)

  - 1 l Superscript Reverse Transcriptase II (200 units/ ul; make sure this is Exonuclease-free)

- Mix gently and incubate at room temperature for 10 min

- Incubate at 42oC for 2-3 hours (no more than ~5 hours; do not do this step overnight)

- Heat sample to 95oC for 2 min

- Place samples on ice, spin down (can store at –20oC at this point if necessary)

# Degrade RNA

- Make sure contents of tubes are spun down

- Add 0.5 l of RNase A (4 mg/ml) at room temperature (Promega)

- Add  0.5 l of RNase H (2 U/l) (Fermentas) (Note:  Not clear if really necessary to use both enzymes; the RNase H is fairly expensive)

- Incubate at 37oC for 15-30 min

(Alternative RNA Degradation)

- Add 3.5 l 0.5 M NaOH/50 mM EDTA, then

- Heat at 65 C, 10 minutes

- Add 5 l 1 M Tris 7.5

Purification (Using Qiagen PCR CleanUp Kit)

- Add 25 l high-quality H2O to samples (add only 17.5 ul if alternative RNA degradation was done)

- Add 2.7 l  3 M Sodium Acetate, pH 5.2

- Add 250 l QIAquick  buffer PB

- Apply each sample to a QIAquick column (the DNA should stick to the column here)

- Centrifuge for 30 sec at full speed

- Take the column flow-through and replace back onto the top of the column and spin a 2nd time

- Place the flow-through back in the original tube and save in case of problems with the purification.

- Wash with 400 l QIAquick buffer PE, spin 30 sec at full speed and discard flow-through (your DNA remains on the column)

- Repeat step 8, discarding flow-through

- Spin the column briefly once more to get rid of remainder of wash solution

- Place column in a clean, well-labeled 1.5 ml elution tube

- Apply 30 l buffer EB to center of column without touching the membrane

- Wait one min, then centrifuge 1 min at full speed (gradually increase from 0 to full speed to avoid shearing off the eppendorf tube lids).  Your DNA is in the flow-through this time!

- Again apply 30 l buffer EB to center of column without touching the membrane.  DNA should be in a volume of 60 l.

# Determining the incorporation of labeled nucleotides

- Use a 384-well spec plate (if available) so that small volumes can be used

- Use undiluted labeled sample and buffer as the blank (~40 l of each).  Read absorbances at 260 nm, 280 nm, 550 nm for Cy3 and 650 for Cy5.

- Calculations:

  - Extinction coefficients are 150,000 for Cy3 and 250,000 for Cy5

  - Purity:  Corrected A260/A280 (want this to be ~1.8)

  - Pmol dye/l of sample = A(dye)/(extinction coefficient *106)

  - dsDNA = 50 g/ml

  - ssDNA = 33 g/ml

  - ssRNA = 40 g/ml

Example:

Total dsDNA (g) = 50 g/ml * 1 ml/l * A260 * Volume (l) * DF

- If labeling efficiencies are fairly close for each dye and the nucleic acid yield is also similar, then about 30 pmol of dye of the sample per hybridization is a good starting point to product good intensity on most arrays.  It is better to control the amounts of starting RNA/cells and have a good internal control and normalization scheme to deal with incorporation discrepancies.

Institute for Systems Biology

Direct Incorporation of Cy3/Cy5 During Reverse Transcription

# Cost Analysis

Reagent	Supplier	Startup Cost		Number of Two	Cost per Two

Label Expts		Label Expt

oligo dT 	Operon		$75 for 100 ug		       	20		$3.75

Superscript II 	Invitrogen 	$220 for 10,000 units	       	25		$8.80

Life Technologies

Cy3–dUTP	Amersham 	25 nmoles for $295	      	25		$11.80

Cy5-dUTP	Biosciences	25 nmoles for $304	      	25		$12.16

Rnase A 	Promega 	$212 - 1 ml, 4 mg/ml		1000		$0.22

Rnase H 	Promega 	$146 for 50 units		25		$5.84

QIAquick 	Qiagen		50 columns for $76		25		$3.04

Totals				$1328  for 25 Experiments		$45.61 per Expt