Thank you for your purchase of our E. coli DNA microarrays. Each EC014 microarray consists of full-length PCR products from all 4,290 annotated open reading frames (ORFs) in E. coli mg1655 (1).  PCR products are printed in replicate on standard, 1” X 3”, poly-l-lysine coated glass microscope slides.  Greater than 95% have been shown to migrate as a single band, of the predicted molecular weight, as determined by gel electrophoresis and ethidium bromide staining.  Also printed on each microarray are >200 control spots consisting of: fragmented E. coli genomic DNA, non-specific salmon sperm DNA, 3X SSC, and >25 different yeast ORF PCR products (see master table). These microarrays are printed using a 16-pin configuration, resulting in a pattern of 32 sub-arrays each having geometry of 17 rows by 18 columns with spot diameter averaging 100 microns. To aid in locating the array, the top and bottom array boundaries are etched on the back of the microscope slide.  A unique serial number is also etched on the array side at the bottom of the slide (see diagram in master table). The integrity and quality of each set of microarrays have been verified by hybridization with fluorescent-labeled cDNA and genomic DNA.  Results from each microarray hybridization will vary from researcher to researcher.

Included with your purchase are a set of protocols used in our lab and a table that identifies genes by their location on the microarray.  Gene product information in the table is based on version M53 of the mg1655 genome sequence (1,2) and is subject to change.  Protocols include E.coli RNA isolation, cDNA and genomic DNA probe labeling, hybridization and post hybridization washing.  Protocols are subject to change.

#### Package Contents

E.coli Genome arrays 	-EC014067-068, 071-084 & EC015007-010, 012-019

# 1 data CD

1.  F. R. Blattner et al,  Science 277, 1453 (1997) “The Complete Genome Sequence of Escherichia coli K-12”.

2. http://www.gcow.wisc.edu/Gec/index.htm